ECA 1011 – MICROPROCESSOR AND MICROCONTROLLERS

Name : Nithish Kumar S

Reg No : 192424021

Exp 01: Analysis Development and Verification of 8 bit and 16 bit Arithmetic Operations using Assembly Language

Objectives:

- To Analysis and Verify 8 bit Addition and Subtraction using 8 bit Micro Processor

- To Analysis and Verify 16 bit Addition and Subtraction using 8 bit Micro Processor

- To Analysis and Verify 8 bit Multiplication and Division using 8 bit Micro Processor

- To Analysis and Verify 16 bit Multiplication and Division using 8 bit Micro Processor

Components Required:

8085 Simulator

Procedure:

- Launch your 8085 Simulator application on your system.

- Type or paste the Assembly Language Program (ALP) into the editor/assembler window.

- Click on the Assemble or Compile button to convert the assembly mnemonics into machine code. Ensure there are no syntax errors.

- Open the Memory Editor window and manually enter your input hex operands into the designated starting memory addresses.

- Click Run or use Step-by-Step execution to trace the accumulation and flag changes.

- Check the specified target memory locations and register statuses to verify your mathematical outputs.

- To Analysis and Verify 8 bit Addition and Subtraction using 8 bit Micro Processor

Code:

Addition:

LDA 4000

MOV B A

LDA 4001

MVI C 00

ADD B

JNC SKIP

INR C

SKIP:

STA 4002

MOV A C

STA 4003

HLT

Data Table:

| 4000 | D5 |
| --- | --- |
| 4001 | A7 |
| 4002 | 7C |
| 4003 | 01 |

Subtraction:

LDA 4000

MOV B A

LDA 4001

MVI C 00

SUB B

JNC SKIP

INR C

CMA

ADI 01

SKIP: STA 4002

MOV A C

STA 4003

HLT

Data Table:

| 4000 | D5 |
| --- | --- |
| 4001 | A7 |
| 4002 | 7C |
| 4003 | 01 |

- To Analysis and Verify 16 bit Addition and Subtraction using 8 bit Micro Processor

Code:

Addition:

MVI C 00

LHLD 4000

XCHG

LHLD 4002

DAD D

JNC SKIP_CARRY

INR C

SKIP_CARRY: SHLD 4004

MOV A C

STA 4006

HLT

Data Table:

| 4000 | D5 |
| --- | --- |
| 4001 | F8 |
| 4002 | CD |
| 4003 | D5 |
| 4004 | A2 |
| 4005 | CD |
| 4006 | 01 |

Subtraction:

MVI C 00

LHLD 4000

MOV A L

LHLD 4002

SUB L

MOV L A

MOV A H

SBB H

MOV H A

JNC SKIP_BORROW

INR C

CMA

MOV H A

MOV A L

CMA

ADI 01

MOV L A

MOV A H

ACI 00

MOV H A

SKIP_BORROW: SHLD 4004

MOV A C

STA 4006

HLT

Data Table:

| 4000 | D5 |
| --- | --- |
| 4001 | F8 |
| 4002 | CD |
| 4003 | D5 |
| 4004 | 08 |
| 4005 | 23 |
| 4006 | 00 |

- To Analysis and Verify 8 bit Multiplication and Division using 8 bit Micro Processor

Code:

Multiplication:

MVI C 00

LDA 4000

MOV B A

LDA 4001

MOV D A

MVI A 00

LOOP: ADD B

JNC SKIP_CARRY

INR C

SKIP_CARRY: DCR D

JNZ LOOP

STA 4002

MOV A C

STA 4003

HLT

Data Table:

| 4000 | 03 |
| --- | --- |
| 4001 | 04 |
| 4002 | 0C |
| 4003 | 00 |

Division:

MVI C 00

LDA 4000

LDA 4001

MOV B A

LDA 4000

LOOP: CMP B

JC STORE

SUB B

INR C

JMP LOOP

STORE: STA 4003

MOV A C

STA 4002

HLT

Data Table:

| 4000 | 0D |
| --- | --- |
| 4001 | 04 |
| 4002 | 03 |
| 4003 | 01 |

- To Analysis and Verify 16 bit Multiplication and Division using 8 bit Micro Processor

Code:

Multiplication:

LHLD 4000

XCHG

LHLD 4002

MOV C L

MOV B H

LXI H 0000

LOOP: MOV A C

ORA B

JZ STORE

DAD D

DCX B

JMP LOOP

STORE: SHLD 4004

HLT

Data Table:

| 4000 | 03 |
| --- | --- |
| 4001 | 04 |
| 4002 | 05 |
| 4003 | 01 |
| 4004 | F0 |
| 4005 | 44 |
| 4006 | 00 |

Division:

LXI B 0000

LHLD 4000

XCHG

LHLD 4002

LOOP: MOV A E

SUB L

MOV A D

SBB H

JC STORE

MOV A E

SUB L

MOV E A

MOV A D

SBB H

MOV D A

INX B

JMP LOOP

STORE: XCHG

SHLD 4006

MOV L C

MOV H B

SHLD 4004

HLT

Data Table:

| 4000 | F4 |
| --- | --- |
| 4001 | 3D |
| 4002 | 05 |
| 4003 | 01 |
| 4004 | 3C |
| 4005 | 00 |
| 4006 | C8 |

Result:

- 8-bit Addition and Subtraction: The target memory location holds the correct 8-bit sum or difference, and the adjacent byte records 01H if an overflow carry or borrow occurred, or 00H if it did not.

- 16-bit Addition and Subtraction: The sequential memory addresses store the low and high bytes of the correct 16-bit sum or difference, along with a final byte reflecting the state of the terminal carry or borrow flag.

- 8-bit Multiplication and Division: The output registers or memory addresses contain either a combined 16-bit product from repeated addition, or two distinct 8-bit bytes representing the exact Quotient and Remainder from repeated subtraction.

- 16-bit Multiplication and Division: The designated target memory blocks accurately hold a multi-byte product (up to 32 bits) generated by the double-addition loop, or a pair of 16-bit values containing the final Quotient and Remainder.